ملخص الرسالة

يتعرض البحث إلى دراسة الخصائص الريولوجية لمربى التين البيوريه. و قد تم فى هذا البحث دراسة العلاقة بين إجهاد القص و معدل الإنفعال لمركزات مربى التين، بغرض الحصول على علاقات يمكن من خلالها التنبؤ بفقد الضغط الناتج عن تدفق هذه المركزات فى مساراتها أثناء التصنيع و قد تم دراسة تأثير العوامل الاتية :                                                                                                         إجهاد القص للمركزات ، معدل الانفعال ، تركيز المربى ، درجة الحرارة ، و من ثم استنتاج  تأثير كل من هذه العوامل على اجهاد الخضوع  ، اللزوجة الظاهرية للمركزات .                                                           و قد تم استنتاج أن هذه المركزات تتبع العلاقة العامة التالية بين معدل الانفعال و اجهاد القص و هى خاصة بموائع بنجهام .

كذلك تم تتبع تصرف منحنيات الأجهاد ، معدل الانفعال عند معدل انفعال ثابت كدالة فى الزمن حيث اتضح وجود ظاهرة الـــ  Thixotropy    و قد تم هذه الظاهرة على ضوء استخدام ثلاثة نماذج رياضية مختلفة .                                                                                                                       تم حساب العلاقة بين معامل الاحتكاك و رقم رينولدز التى يمكن من خلالها استنتاج الهبوط فى الضغط المصاحب لمثل هذه المركزات

تم حساب قوة المضخة اللازمة لضخ المركز لماكينة التعبئة  و وجد أنها تساوى 0.38 حصان

Summary

The Rheological properties of Fig Jam puree were studied over the range: 20-90°C, concentrations 40-65% and the speed of spindle 10-50 rpm. Shear stress- Shear rate data indicate that the puree behaves as a non-Newtonian Bingham plastic fluid with yield stress.

The yield stress decreases with increasing temperature except for the samples that have solid concentrations from 60-65% wt. due to pectin formation. Also, the yield stress increases with an increase in solid concentrations (at all temperatures investigated). Fig jam puree exhibits a thixotropic behavior in which, the apparent viscosity of the material decreases with time of shearing at constant shear rate.

Three models were applied to simulate thixotropic behavior. Dependence of apparent viscosity on temperature was related through the Arrhenius law and the plot of ln(µ) versus (1/T) exhibits three regions with different temperature dependencies. Also, this study includes the dependence of the apparent viscosity of Fig jam puree on solid concentration, that follows the power law relationship.

The generalized friction factor-Reynolds number was constructed taking in consideration the results at different shear rates and concentrations.

دراسة الخـواص اللانيـوتـونية لمركزات الفاكهة

إعداد

منال عبد الرحمن سرور

رسالة مقدمة إلى كلية الهندسة، جامعة القاهرة

كجزء من متطلبات الحصول على درجة الدكتوراة

فى الهندسة الكيميائية

تحت إشراف

أ.د. مجدى فؤاد أبادير						أ.د.لطفى عبد الحميد حبش

أستاذ التكنولوجيا غير العضوية					     أستاذ الصناعات الغذائية

كلية الهندسة - جامعة القاهرة					     مركز البحوث الزراعية

قسم الهندسة الكيميائية						معهد بحوث تكنولوجيا الأغذية

د. سلوى رأفت مصطفى

أستاذ مساعد

كلية الهندسة - جامعة القاهرة

قسم الهندسة الكيميائية

كلية الهندسة - جامعة القاهرة

2005

Study of the Non-Newtonian Properties of fruit concentrates

By

Manal Abdel Rahman Ali Sorour

A Thesis Submitted to the

Faculty of Engineering at Cairo University

In Partial Fulfillment of the Requirements for the Degree of

Doctor of Philosophy

In

Chemical Engineering

Under the supervision of

Prof. Dr. Magdi Fouad Abadir

Professor of Inorganic Technology

Cairo University, Chem. Eng. Dept.

Prof. Dr. Lotfi Abdel Hamid Habash

Professor of food Technology

Agric. Research center

Food Technology Institute

Dr. Salwa Raafat Mostafa

Cairo University, Chem. Eng. Dept.

Faculty of Engineering at Cairo University

2005